# Designing and Analyzing an Industrial Power Distribution System with ETAP

In industrial facilities, reliable power distribution is not just a convenience — it’s the backbone of operations. Any fault or instability can shut down critical equipment, disrupt production, and even compromise safety. To avoid such risks, engineers use tools like ETAP to design and simulate power networks before they are deployed.

This article explores a Single Line Diagram (SLD) created in ETAP, representing a multi-level industrial electrical distribution system with redundancy, step-down transformation, and diverse load connections.

## High-Level Overview of the System

The SLD begins with a high-voltage utility source (125 kV, 1253 MVA) feeding into the industrial network. From there, the design includes multiple transformers (TR1, TR3, TR5, TR7, TR10) that step down power across different voltage levels, ensuring that loads from heavy equipment to sensitive electronics are powered correctly.

Key highlights:

- Incoming Source: PowerGrid 1 (125 kV, 1253 MVA)

- Main Bus: Distributes power to multiple transformers and feeders

- Step-Down Transformers: Voltage levels are progressively reduced from 125 kV down to 0.49 kV

- Loads Connected: Pumps, lighting, UPS, and other critical industrial equipment

## System Components

### 1. Utility Supply & Main Bus

- The incoming utility is represented at 125 kV and tied to the bus through circuit breakers (CBs).

- This bus acts as the central hub, distributing power to multiple transformers and feeders.

### 2. Transformers & Voltage Levels

The system cascades through different transformers to supply the appropriate voltage to equipment:

- TR1: Steps down 125 kV to medium-voltage distribution.

- TR5: Supplies 13.2 kV, 20 MVA for industrial motors and pumps.

- TR7 / TR10: Provide 10.48 kV and 0.49 kV for mixed loads.

- TR64: Dedicated 7.5 kVA, 0.49 kV low-voltage supply for smaller loads and UPS backup.

### 3. Loads

- Pump A (100 kVA): Represents heavy mechanical equipment.

- Lighting (100 kVA): Facility-wide illumination.

- UPS (Uninterruptible Power Supply): Ensures critical equipment runs smoothly during disturbances.

- Auxiliary loads (100 kVA–1000 kVA range): Balance between process equipment and support systems.

## Features & Capabilities Modeled in ETAP

Using this model, ETAP can simulate:

- Load Flow Analysis → Evaluates how power is distributed across buses and transformers.

- Short-Circuit Studies → Determines fault current levels and helps size breakers.

- Relay Coordination → Ensures selective isolation of faults.

- Arc Flash Studies → Identifies incident energy levels for workplace safety.

- Contingency Scenarios → Tests redundancy and response to utility or transformer outages.

## Why This Matters in Industry

Industrial plants often operate with tiered loads:

- Heavy machinery (pumps, motors, crushers)

- Essential plant services (lighting, HVAC)

- Critical IT infrastructure (servers, control systems, UPS)

Without a properly designed distribution system, power quality issues or a single fault could cascade into full plant downtime. This ETAP model ensures that:

- Each load gets the correct voltage level.

- Redundancy and backup systems are in place.

- Safety is prioritized via protection and arc-flash coordination.

## Conclusion

This ETAP-based SLD demonstrates how complex industrial networks can be visualized, analyzed, and optimized before commissioning. From 125 kV utility interconnection down to 480 V distribution, the system balances heavy industrial loads with sensitive IT equipment, ensuring both reliability and safety.

As industries move towards digital transformation and smart grids, models like this will be the foundation for integrating renewables, energy storage, and advanced automation into existing networks.

🚀 Reliable power isn’t just about keeping the lights on — it’s about ensuring that every motor, pump, and server in a facility works seamlessly together.